Q. Demonstrate DHCP Server in Cisco Packet Tracer.

Aim: To configure a router as DHCP server to automatically assign IP addresses to clients.

Devices Required:

1 Router (1841 or any model)

1 Switch (2960 or any model)

2-4 PCs Connections:

Router FastEthernet0/0 to Switch FastEthernet0/1 (Straight-through cable)

PC0 to Switch FastEthernet0/2 (Straight-through cable)

PC1 to Switch FastEthernet0/3 (Straight-through cable) Topology :

Code :

Router Configuration Commands:

Router>enable

Router#configure terminal

Router(config)#hostname DHCP-Router

DHCP-Router(config)#interface FastEthernet0/0

DHCP-Router(config-if)#ip address 10.0.0.1 255.255.255.0

DHCP-Router(config-if)#no shutdown

DHCP-Router(config-if)#exit

DHCP Pool Configuration:

DHCP-Router(config)#ip dhcp pool STUDENT_NETWORK

DHCP-Router(dhcp-config)#network 10.0.0.0 255.255.255.0

DHCP-Router(dhcp-config)#default-router 10.0.0.1

DHCP-Router(dhcp-config)#dns-server 8.8.8.8

DHCP-Router(dhcp-config)#exit

Exclude IP Addresses:

DHCP-Router(config)#ip dhcp excluded-address 10.0.0.1 10.0.0.10

DHCP-Router(config)#exit

DHCP-Router#copy running-config startup-config

PC Configuration:

On each PC, go to Desktop tab → IP Configuration

Select DHCP option Verification cammand :

On Router:

DHCP-Router#show ip dhcp pool

DHCP-Router#show ip dhcp binding

DHCP-Router#show ip interface brief

On  both PCs (Command Prompt):

C:\>ipconfig

C:\>ipconfig /all

C:\>ping 10.0.0.1

Output Results:

PC0 - ip config:

PC1 - ip config:

Router - DHCP Bindings (WORKING COMMAND):

Router - DHCP Pool Status (WORKING COMMAND):

Router - Interface Status:

Connectivity Test - PC0 to Router:

Conclusion: The DHCP server has been successfully configured on the router using commands compatible with Cisco Packet Tracer.